- SCOPE OF WORK

# SUPPLY OF AND MAINTAINING STRINGING EQUIPMENT FOR MINERVA-VULCAN 400KV LINE RESTORATION PROJECT Eskom Rotek Industries require the following for MINERVA-VULCAN 400KV LINE RESTORATION project as and when required. Supply and maintain and deliver stringing machines as and when required: Model Number; Description. SPW19G; Puller Machine 19000kg's complete with remote control and data recording, Maximium Pulling Speed of 5km's per Hour Weight 5100kg's. B15009x2G; Tensioner Machine 18000kg's, Split Bull Wheel configuration for independent operation or in fixed mode if required. Maximum of 6 Conductors Weight 8300kg's. 770852G; Suitable to connect 3 conductors to the pulling rope, with balancing wheels with counter weights. 772076G; 900mm nylon wheel x 3 complete assembly. TBIT7G; Galvanised Steel Frame suitable for Wooden drums 7000kg's. TBITMOTG; Motor Hydraulic complete with Hose assembly. 601024G; Braided Steel Rope high tensile Steel on reel with Eyes both ends.

Compiled by:                                                                            Supported by:

James Pohl        	    				         KevinMuirhead                                          	Acting Project manager             	                                    Senior Quantity Surveyor

Construction Services	                                                                    Construction Service

Supported by:

John Tshabalala

Acting Powerlines Portfolio manager

Construction Services