KEY TO DESCRIPTORS USED IN PREPARING THE CORE LOGS

Colour

Colour is a useful guide to rock composition and state of weathering.    As with soils, the colouring is often complex and variations may be introduced by banding, veining, staining of fracture faces, mottling, etc.

Weathering

Unweathered - No visible signs of alteration in the rock material, but fracture planes may be stained or discoloured.

Slightly weathered – Fractures are stained or discoloured and may contain a thin filling of altered or illuviated material.   Discoloration may extend into the rock from the fracture planes to a distance of up to 20 per cent of the fracture spacing (i.e. less than 40 per cent of the core is discoloured).

Moderately weathered – Slight discoloration extends from the fracture planes for a distance greater than 20 per cent of the fracture spacing (i.e. through the greater part of the rock).   Fractures may contain filling of altered or illuviated material.   The surface of the rock is not friable (except in the case of poorly cemented sedimentary rocks) and the original texture of the rock is preserved.   Grain boundaries are sometimes partially open.

Highly weathered – Discoloration extends throughout the rock, and the surface is friable.   In rock cores the surface is usually pitted due to the washing out of highly altered minerals by the drilling fluid.   The original texture of the rock has generally been preserved but separation of grains has occurred.

Completely weathered – The rock is totally discoloured and the external appearance of excavated blocks or lengths of core is that of a residual soil.   Internally the rock texture is partly preserved but grains have separated completely.   Such residual material should be described in terms of the normal soil descriptors MCCSSO.

Texture

In most soils aggregates of particles can be broken down between the fingers without difficulty and a tactile assessment of the dominant particle sizes can be made;  this is not possible with rock and a field description must be based on a visual assessment.   Experience indicates that this kind of appraisal tends to bias the average texture in favour of the coarser particle sizes.   The following descriptors and class intervals have been selected with this bias in mind so as to conform in a general way with the principal classes used in the engineering description of soils:

Very coarse:	grains or crystals >6.0 mm

Coarse:	grains or crystals 2.0 – 6.0 mm

Medium:	grains or crystals 0.6 – 2.0 mm

Fine:		grains or crystals 0.2 – 0.6 mm

Very fine:	grains or crystals <0.2 mm

Microstructure and fracture spacing

Bedding plane spacing – Under this heading are also included foliation in metamorphic rocks and flow banding in igneous rocks:

Very thickly bedded	>2.0 m

Thickly bedded	0.6 – 2.0 m

Medium bedded	0.2 – 0.6 m

Thinly bedded		60 mm – 0.2 m

Very thinly bedded	20 – 60 mm

Laminated		6 – 20 mm

Thinly laminated	<6 mm

For the purposes of this system of description fractures include all mechanical discontinuities such as joints, faults, shear-zones, bedding planes, foliation, and cleavage planes and flow bands, if these represent planes of weakness.  The spacing is measured normal to the inclination of the fractures or the various fracture sets.

Fracture spacing

Fractures per m

Very slightly fractured	>2.0 m			<1

Slightly fractured		0.6 – 2.0 m		about 1

Moderately fractured		0.2 – 0.6 m		1 – 5

Highly fractured		20 mm – 0.2 m	5 – 50

Very highly fractured		>20 mm		>50

Hardness

The various categories of rock hardness are identified as follows:

Very soft rock – Material crumbles under firm blows with the sharp end of geological pick and can be peeled with a knife;  too hard to cut an undisturbed sample by hand.   Typical unconfined compressive strength (UCS):  700 – 4000 kPa.

Soft rock – Can just be scraped and peeled with a knife;  indentations 1 – 3 mm deep are formed in the specimen by firm blows of the pick point.   Typical UCS:  4000 – 10 000 kPa.

Moderately hard rock – Cannot be scraped or peeled with a knife.   Hand-held specimen can be broken with a single firm blow of the hammer end of a geological pick.   Typical UCS:  10 000 – 20 000 kPa.

Hard rock – Typical UCS:  20 000 – 70 000 kPa

Very hard rock – Typical UCS:  70 000 – 200 000 kPa

Extremely hard rock – >200 000 kPa

For these three categories, many blows needed to break specimen.   Point load tests, verified by laboratory UCS test, needed to distinguish categories.

Rock type

The rock type should be identified in terms of the criteria given for various categories of igneous, metamorphic, and sedimentary rocks in Chapter 3.   Like the origin of a soil, the stratigraphic horizon to which a rock belongs often provides a useful guide to the engineering properties of the rock mass, and should be identified wherever possible.

DESCRIPTION OF THE FRACTURE SURFACES

Fracture type

The main fracture types are listed above.   The identification of the origin of the fracture is important in assessing its engineering significance.

Separation of faces

This has a marked effect on the strength of the rock mass:

Closed		0 mm

Very narrow	0 – 0.6 mm

Narrow	0.6 – 2.0 mm

Wide		2.0 – 6.0 mm

Very wide	6.0 – 20 mm

Major fracture >20 mm

Staining or filling

These are identified as follows:

Clean:  no filling material

Stained:  coloration of fracture faces only.   No recognizable filling material

Filled:  recognizable filling material

Smooth:  appears smooth and is generally smooth to the touch.   May be slickensided (record direction).

Slightly rough:  asperities on the fracture surfaces are visible and can be distinctly felt.

Medium rough:  asperities are clearly visible and fracture surface feels abrasive.

Rough:  large angular asperities are visible, some in the form of ridges and steps with high side angles.

Very rough:  near vertical steps and ridges occur on the fracture surface.

Orientation

In unorientated cores the relative dip of the various fracture sets in relation to the horizontal is recorded.

DESCRIPTION OF THE FRACTURE FILLING

Fracture fillings often influence the strength of the rock mass, and the extent of this influence is often predictable from a proper description.   Identification of the type of filling is often useful, e.g.. iron oxides, calcite, chlorite, silt or clay, siliceous material, etc.

Borehole and core logs

In addition to the above descriptors, a core log prepared from borehole samples usually includes several additional parameters:

Core recovery – The percentage of each core (i.e. the distance over which the borehole was deepened between withdrawing the core-barrel and extracting the core) which is represented by solid material.   Where losses of core are recorded, their precise position should, whenever possible, be indicated on the core log.

Rock quality designation (RQD) – This is the percentage of the length of core recovered in a drill-run which is in the form of pieces of fully circular core, each with a length of more than 100 mm.   Some important rock mechanics indices can be correlated with the RQD value.

Fracture frequency – This is the number of fractures which intersect a unit length of rock (either a length of a core or of a particular axis selected to traverse the rock mass).